SMART CRICKET: AI-Based Batting Shot Detection and Real-Time Feedback System

A PROJECT REPORT

###### Submitted by

Dibyasha Mishra 24BAI10223

Sakshee Kumari 24BAI10384

Abhijai Raghuvanshi 24BAI10024

Shambhavi Singh 24BAI10077

Khushi khatri 24BAI10311

RAMANADULA SHYAM DANIEL RAJ 24BAI10847

in partial  fulfillment  for the  award  of  the  degree

of

##### BACHELOR OF TECHNOLOGY

in

COMPUTER SCIENCE AND ENGINEERING

(Specialization in Artificial Intelligence and Machine Learning)

SCHOOL OF COMPUTING SCIENCE AND ENGINEERING

VIT BHOPAL UNIVERSITY

KOTHRIKALAN, SEHORE

MADHYA PRADESH - 466114

April & 2026

VIT BHOPAL UNIVERSITY, KOTHRIKALAN, SEHORE

MADHYA PRADESH – 466114

BONAFIDE CERTIFICATE

Certified that this project report titled “Smart Cricket: AI-Based Batting Shot Detection and Real-Time Feedback System” is the bonafide work of                                                 Dibyasha Mishra 24BAI10223, Sakshee Kumari 24BAI10384, Abhijai Raghuvanshi 24BAI10024, Shambhavi Singh 24BAI10077, Khushi khatri 24BAI10311, RAMANADULA SHYAM DANIEL RAJ 24BAI10847 who carried out the project work under my supervision. Certified further that to the best of my knowledge, the work reported here does not form part of any other project/research work based on which a degree or award was conferred on an earlier occasion on this or any other candidate.

PROJECT SUPERVISOR

Dr. Shiv Shankar Prasad Shukla

School of Computer Science and Engineering

VIT BHOPAL UNIVERSITY

The Project Exhibition II Examination is held on ___________

ACKNOWLEDGEMENT

I would like to express my sincere gratitude to all those who have supported and guided me throughout the completion of this project titled “Smart Cricket: AI-Based Batting Shot Detection and Real-Time Feedback System.”

I am deeply thankful to the Head of the Department _________________, School of Computing Science and Engineering, for providing the necessary facilities and encouragement to carry out this work successfully.

I would like to express my heartfelt gratitude to my project guide Dr. Shiv Shankar Prasad Shukla, for their continuous support, valuable suggestions, and guidance throughout the project. Their insights and encouragement played a crucial role in shaping this work.

I also extend my thanks to all the faculty members and technical staff of the department for their assistance and support.

Finally, I am extremely grateful to my parents and friends for their constant encouragement, motivation, and support during the completion of this project.

LIST OF ABBREVIATIONS

| ABBREVIATION | FULL FORM |
| --- | --- |
| AI | Artificial Intelligence |
| CV | Computer Vision |
| ML | Machine Learning |
| OpenCV | Open Source Computer Vision Library |
| RF | Random Forest |
| FPS | Frames Per Second |
| UI | User Interface |
| JSON | JavaScript Object Notation |
| CSV | Comma Separated Values |
| 2D | Two Dimensional |
| 3D | Three Dimensional |
| RGB | Red Green Blue |
| API | Application Programming Interface |
| GUI | Graphical User Interface |

LIST OF FIGURES

| FIGURE NO. | TITLE | PAGE NO. |
| --- | --- | --- |
| 1 | System Architecture |  |
| 2 | Pose Detection Output |  |
| 3 | Keypoint Extraction |  |
| 4 | Shot Prediction Screen |  |
| 5 | Feedback Interface |  |
| 6 | Performance Graph |  |

LIST OF TABLES

| TABLE NO. | TITLE | PAGE NO. |
| --- | --- | --- |
| 5.1 | Model Performance | 19 |

ABSTRACT

This project presents an intelligent system titled “Smart Cricket: AI-Based Batting Shot Detection and Real-Time Feedback System”, which aims to assist cricket players in improving their batting techniques using computer vision and machine learning.

The system is designed to recognize different cricket shots such as cover drive, pull shot, hook shot, and sweep shot by analyzing body posture through pose detection. A dataset was created by collecting video footage of professional cricket players performing various shots. Frames were extracted from these videos and processed using pose estimation techniques to obtain key body landmarks.

A machine learning model was trained on this dataset to classify batting shots with an accuracy of approximately 89%. The trained model is integrated with a real-time computer vision system that captures live video input from the user. The system detects the user's pose, predicts the shot being played, and compares the user’s joint angles (such as elbow and knee angles) with predefined ideal angles derived from professional players.

Based on this comparison, the system provides real-time feedback to the user, such as correcting posture and improving technique. Additionally, a scoring mechanism is implemented to evaluate the user’s performance. The project also includes 3D modeling of cricket shots to enhance visualization and understanding.

This system demonstrates the potential of artificial intelligence in sports training by providing an affordable and accessible virtual coaching solution.

TABLE OF CONTENTS

| CHAPTER NO. | TITLE | PAGE NO. |
| --- | --- | --- |
|  | List of Abbreviations List of Figures List of Tables Abstract | iii iv v vi |
| 1 | INTRODUCTION # Introduction 1.2 Motivation for the work 1.3 Overview 1.5 Problem Statement 1.6 Objective of the work 1.7 Organization of the thesis 1.8 Summary | 1-3 |
| 2 | LITERATURE SURVEY 2.1 Introduction 2.2 Core area of the project 2.3 Existing Algorithms 2.3.1 Algorithm1 2.3.2 Algorithm2 2.3.3 Algorithm3 2.4 Other method used in the project 2.5 Research issues/observations from literature Survey 2.6 Summary | 4-7 |
| 3 | SYSTEM ANALYSIS 3.1 Introduction 3.2 Disadvantages/Limitations in the existing system 3.3 Proposed System 3.4 Summary | 8-12 |
| 4 | SYSTEM DESIGN AND IMPLEMENTATION 4.1 Introduction 4.2 Module 1 design & implementation 4.3 Module 2 design & Implementation 4.5 Summary | 13-17 |
| 5 | PERFORMANCE ANALYSIS 5.1 Introduction 5.2 Performance Measures 5.3 Performance Analysis # 5.4 Summary | 18-19 |
| 6 | # FUTURE ENHANCEMENT AND CONCLUSION 6.1 Introduction 6.2 Limitation/Constraints of the System 6.3 Future Enhancements 6.4 Conclusion | 20-21 |
|  | # Appendix A # Appendix B # References | 22 23 24 |

CHAPTER 1: INTRODUCTION

1.1 Introduction

Cricket is one of the most popular sports worldwide, and batting technique plays a crucial role in determining a player's performance. Traditionally, coaching in cricket relies heavily on human observation and experience, which may not always provide precise and consistent feedback. With the advancement of artificial intelligence and computer vision, it is now possible to analyze human movements in real time and provide data-driven insights.

This project, titled “Smart Cricket: AI-Based Batting Shot Detection and Real-Time Feedback System”, focuses on developing an intelligent system that can detect cricket shots and provide instant feedback to players. The system uses computer vision techniques to capture human body movements and machine learning algorithms to classify different types of batting shots.

By leveraging pose detection technology, the system extracts key body landmarks and analyzes joint movements. A trained machine learning model is used to identify the type of shot played by the user. Furthermore, the system compares the user’s posture with ideal professional standards and provides corrective suggestions. This enables players to improve their technique without requiring constant supervision from a coach.

1.2 Motivation for the work

The motivation behind this project is to bridge the gap between professional cricket coaching and accessibility. Not all aspiring players have access to expert coaching due to financial or geographical constraints. Additionally, manual coaching may sometimes lack precision and consistency.

With the rise of artificial intelligence, there is an opportunity to develop smart systems that can provide personalized and real-time feedback. This project aims to create an affordable and automated cricket coaching solution that helps players analyze their performance and improve their skills effectively.

1.3 Overview of the Project

The proposed system integrates MediaPipe-based pose detection with a Random Forest machine learning model to identify cricket shots and evaluate player posture.

The workflow of the system includes:

- Collecting video data of professional cricket players

- Extracting frames and body landmarks using pose detection

- Training a Random Forest classifier to recognize different shots

- Capturing live video input from the user

- Predicting the shot in real time

- Calculating joint angles such as elbow and knee angles

- Comparing these angles with ideal values

- Providing corrective feedback and performance score

The system also incorporates visualization techniques, including skeletal tracking and on-screen feedback, to enhance user interaction.

1.4 Problem Statement

Cricket players, especially beginners, often struggle to identify and correct mistakes in their batting technique due to lack of immediate and precise feedback. Traditional coaching methods may not always be accessible, consistent, or data-driven.

Therefore, there is a need for an intelligent system that can automatically detect cricket shots, analyze body posture, and provide real-time corrective feedback to improve batting performance.

1.5 Objectives of the Work

The main objectives of this project are:

- To develop a system capable of recognizing different cricket batting shots

- To implement pose detection for extracting human body landmarks

- To train a Random Forest model for shot classification

- To analyze joint angles and compare them with ideal standards

- To provide real-time feedback and performance scoring

- To create an interactive and user-friendly interface for training

1.6 Organization of the Report

This report is organized as follows:

- Chapter 1 presents the introduction, motivation, and objectives of the project

- Chapter 2 discusses the literature survey and existing approaches

- Chapter 3 explains the system analysis and proposed methodology

- Chapter 4 describes system design and implementation

- Chapter 5 presents performance analysis and results

- Chapter 6 concludes the project and suggests future enhancements

1.7 Summary

This chapter introduced the concept of an AI-based cricket coaching system and highlighted the motivation, objectives, and overall structure of the project. The following chapters will provide detailed insights into the methodologies and implementation of the system.

CHAPTER 2: LITERATURE SURVEY

2.1 Introduction

With the rapid advancement of artificial intelligence and computer vision, significant research has been conducted in the field of human activity recognition and sports analytics. These technologies enable machines to understand and interpret human movements, making them highly useful in applications such as fitness tracking, rehabilitation, and sports training.

In cricket, analyzing batting techniques traditionally depends on expert coaches. However, recent developments have introduced automated systems that use pose estimation and machine learning to evaluate player performance. This chapter reviews existing methods and technologies relevant to the proposed system.

2.2 Core Area of the Project

The core area of this project includes:

- Computer Vision

- Human Pose Estimation

- Machine Learning for Classification

Pose estimation is used to extract body landmarks, and machine learning models are used to classify cricket shots.

Human pose estimation is a technique used to detect and track key points of the human body, such as joints and limbs, from images or videos. Modern pose estimation frameworks provide accurate real-time tracking, making them suitable for sports analysis.

One of the widely used frameworks is MediaPipe Pose, which provides high-performance real-time pose detection. It identifies multiple body landmarks such as shoulders, elbows, hips, knees, and ankles. These landmarks can be used to analyze posture, calculate joint angles, and study movement patterns.

Pose estimation plays a crucial role in this project, as it forms the foundation for extracting meaningful features from cricket shots.

Machine learning algorithms are widely used for classifying human activities based on extracted features. In sports analytics, these models help identify different actions such as running, jumping, or specific sports movements.

Among various algorithms, the Random Forest classifier is a popular choice due to its robustness, accuracy, and ability to handle high-dimensional data. It works by constructing multiple decision trees and combining their outputs to improve prediction accuracy.

In this project, the Random Forest model is trained on pose-based features extracted from cricket shot videos to classify different batting techniques.

Computer vision has been increasingly used in sports to enhance performance analysis and coaching. Applications include motion tracking, player performance evaluation, and automated feedback systems.

In cricket, computer vision techniques have been applied to:

- Detect player movements

- Analyze batting posture

- Track ball trajectory

- Evaluate shot execution

These systems provide objective and data-driven insights, reducing dependency on manual observation.

2.3 Existing Algorithms

2.3.1 Convolutional Neural Networks (CNN)

CNNs are widely used for image classification and activity recognition. They automatically extract spatial features from images and can be used to identify complex patterns in human movements.

However, CNNs require large datasets and high computational power, making them less suitable for lightweight real-time systems.

2.3.2 Support Vector Machine (SVM)

SVM is a supervised machine learning algorithm used for classification tasks. It works by finding the optimal hyperplane that separates different classes.

SVM performs well on smaller datasets but may not scale efficiently with large pose-based feature sets.

2.3.3 Random Forest Algorithm

Random Forest is an ensemble learning method that combines multiple decision trees to improve accuracy and reduce overfitting.

Advantages:

- High accuracy

- Handles high-dimensional data

- Works well with structured features like pose keypoints

In this project, Random Forest is used for cricket shot classification and achieves an accuracy of approximately 84%.

2.4 Other Methods Used in the Project

Apart from machine learning algorithms, the system uses:

- MediaPipe Pose for real-time pose detection

- OpenCV for video processing

- Angle calculation techniques for posture analysis

2.5 Observations from Literature Survey

- Pose estimation is essential for analyzing human movement

- Machine learning models can effectively classify sports actions

- Existing systems lack real-time feedback mechanisms

- There is a need for integrated systems combining detection and feedback

2.7 Summary

This chapter reviewed existing algorithms such as CNN, SVM, and Random Forest. It also highlighted their advantages and limitations. The proposed system uses Random Forest due to its efficiency and accuracy.

CHAPTER 3: SYSTEM ANALYSIS

3.1 Introduction

This chapter presents the analysis of the existing systems and introduces the proposed Smart Cricket system. It highlights the limitations of traditional cricket coaching methods and explains how the proposed system overcomes these challenges using computer vision and machine learning techniques.

The system focuses on detecting cricket shots and providing real-time feedback by analyzing player posture using pose estimation.

3.2 Disadvantages / Limitations of Existing Systems

- Traditional and existing systems for cricket training have several limitations:

- Dependence on human coaches: Players require expert supervision, which may not always be available

- Lack of real-time feedback: Players do not receive immediate correction during practice

- Subjective analysis: Coaching feedback may vary from person to person

- Limited use of technology: Many systems do not use AI-based analysis

- High cost: Advanced training systems are expensive and not accessible to all players

- No posture comparison: Existing systems rarely compare user posture with ideal professional standards

3.3 Proposed System

The proposed system is an AI-based Smart Cricket Coaching System that uses pose detection and machine learning to detect cricket shots and provide real-time feedback.

The system performs the following functions:

- Captures video input using a webcam

- Detects human body pose using MediaPipe

- Extracts keypoints from body landmarks

- Classifies cricket shots using a Random Forest model

- Calculates joint angles (elbow and knee)

- Compares user posture with ideal angles

- Provides corrective feedback

- Displays performance score

3.3.1 System Architecture

The system architecture consists of two main phases:

1. Training Phase

- Collection of cricket shot videos

- Frame extraction from videos

- Pose detection using MediaPipe

- Feature extraction (keypoints)

- Training Random Forest model

2. Testing (Real-Time) Phase

- Capture live video using webcam

- Perform pose detection

- Extract keypoints

- Predict shot using trained model

- Calculate joint angles

- Compare with ideal dataset

- Generate feedback and score

    - Functional Modules

3.3.2.1 Data Collection and Dataset Creation

- Videos of professional players are collected

- Shots include:

  - Cover Drive

  - Straight Drive

  - Pull Shot

  - Hook Shot

  - Sweep Shot

  - Helicopter Shot

  - Paddle Sweep

  - Upper Cut

  - Reverse Sweep

- Frames are extracted and labeled accordingly

3.3.2.2 Pose Detection Module

- Uses MediaPipe Pose

- Detects body landmarks such as:

  - Shoulder

  - Elbow

  - Wrist

  - Hip

  - Knee

  - Ankle

- Each landmark contains:

  - x, y, z coordinates

  - visibility score

3.3.2.3 Feature Extraction Module

- Extracts keypoints from pose landmarks

- Converts them into feature vectors

- These features represent body posture

- Used as input for machine learning model

3.3.2.4 Machine Learning Module

- Uses Random Forest Classifier

- Trained on pose-based features

- Performs multi-class classification of cricket shots

- Achieves approximately 89% accuracy

3.3.2.5 Shot Prediction Module

- Takes real-time input from webcam

- Predicts shot type

- Uses smoothing technique (deque) to stabilize predictions

- Outputs:

  - Shot name

  - Confidence score

3.3.2.6 Angle Calculation Module

- Calculates joint angles using mathematical formulas

- Important angles:

  - Elbow angle (shoulder–elbow–wrist)

  - Knee angle (hip–knee–ankle)

- Uses vector-based angle calculation

3.3.2.7 Feedback Generation Module

- Compares user angles with ideal dataset

- Provides feedback such as:

  - “Raise Elbow”

  - “Bend Knee”

- Determines correctness using tolerance values

3.3.2.8 Scoring Module

- Calculates performance score based on angle deviation

- Score is displayed as percentage

- Helps users track improvement

3.3.2.9 Visualization Module

- Displays:

  - Live video feed

  - Skeleton tracking

  - Highlighted joints

  - Shot name

  - Confidence

  - Feedback

  - Score

3.4 Summary

This chapter analyzed the limitations of existing systems and introduced the proposed AI-based Smart Cricket system. It explained the system architecture and functional modules in detail. The next chapter will describe the system design and implementation.

CHAPTER 4: SYSTEM DESIGN AND IMPLEMENTATION

4.1 Introduction

This chapter describes the design and implementation of the Smart Cricket system. It explains how different modules are integrated to perform cricket shot detection and provide real-time feedback using computer vision and machine learning techniques.

The system is implemented using Python and integrates technologies such as MediaPipe for pose detection, OpenCV for video processing, and a Random Forest classifier for shot prediction.

4.2 System Design

The system is designed as a pipeline where input data flows through multiple stages to generate the final output. It consists of both training and real-time execution phases.

Overall Design Flow

Training Phase:

- Video Collection → Frame Extraction → Pose Detection → Feature Extraction → Model Training

Real-Time Phase:

- Webcam Input → Pose Detection → Feature Extraction → Shot Prediction → Angle Calculation → Feedback Generation → Display Output

Flowchart Steps

- Start

- Capture frame from webcam

- Detect pose using MediaPipe

- Extract keypoints

- Predict shot

- Calculate joint angles

- Compare with ideal values

- Generate feedback

- Display output

- Repeat

4.3 Module Design and Implementation

4.3.1 Module 1: Data Collection and Preprocessing

A dataset was created by collecting videos of professional cricket players performing different batting shots such as cover drive, pull shot, hook shot, helicopter shot, and reverse sweep.

Frames were extracted from these videos and processed using pose detection. Each frame was labeled according to the type of shot, forming a structured dataset for training.

4.3.2 Module 2: Pose Detection using MediaPipe and Feature Extraction

The system uses MediaPipe Pose to detect human body landmarks in real time. It extracts keypoints such as:

- Shoulder

- Elbow

- Wrist

- Hip

- Knee

- Ankle

Each landmark contains coordinates (x, y, z) and visibility score. These keypoints are used as input features for the machine learning model.

Pose detection is performed using MediaPipe Pose, which detects body landmarks in real time.

Each landmark provides:

- x-coordinate

- y-coordinate

- z-coordinate

- visibility

These landmarks are combined to form feature vectors that represent the player's posture. These features are used as input to the machine learning model.

4.3.3 Module 3: Machine Learning Model Implementation

A Random Forest classifier is used to classify cricket shots based on extracted pose features.

Steps:

- Train model using dataset

- Store model using Joblib

- Load model during real-time execution

The model predicts the shot class and outputs a confidence score.

4.3.4 Module 4: Real-Time Shot Prediction

The trained model is integrated with a webcam to perform real-time prediction.

- Frames are captured continuously

- Pose landmarks are extracted

- Features are passed to the model

- Shot is predicted

To improve prediction stability, a smoothing technique using a queue (deque) is applied.

4.3.5 Module 5: Angle Calculation

Joint angles are calculated to analyze player posture. The angle between three points is calculated using vector mathematics:

- Elbow Angle = angle between shoulder, elbow, wrist

- Knee Angle = angle between hip, knee, ankle

These angles are essential for evaluating correctness of posture.

4.3.6 Module 6: Feedback and Scoring System

The calculated angles are compared with ideal angles stored in a dataset.

Feedback Rules:

- If angle is within tolerance → Correct

- If angle deviates → Suggest correction

Examples:

- “Raise Elbow”

- “Bend Knee”

Scoring:

- Each joint is scored

- Final score is average

- Displayed as percentage

4.3.7 Module 7: Visualization and User Interface

The system provides an interactive interface using OpenCV.

Features include:

- Live video display

- Skeleton tracking

- Highlighted joints (correct/incorrect)

- Shot name and confidence

- Feedback messages

- Performance score

This enhances user experience and makes training effective.

4.4 Tools and Technologies

- Programming Language: Python

- Pose Detection: MediaPipe

- Machine Learning: Random Forest (Scikit-learn)

- Computer Vision: OpenCV

- Data Processing: NumPy, JSON

- Model Storage: Joblib

4.5 Summary

This chapter described the complete design and implementation of the seven modules of the AI-Based Cricket Batting Stance Evaluation System. The pipeline integrates live webcam capture, real-time ML-based shot classification, dynamic stance selection, joint angle computation, directional corrective feedback, and a color-coded live video overlay. The use of a pre-trained model and deque-based smoothing distinguishes this implementation from simple rule-based approaches and ensures robust, temporally stable performance.

CHAPTER 5: PERFORMANCE ANALYSIS

5.1 Introduction

This chapter presents the performance evaluation of the proposed Smart Cricket system. It analyzes the effectiveness of the machine learning model in classifying cricket shots and evaluates the real-time feedback mechanism.

The performance of the system is measured using various parameters such as accuracy, response time, and feedback correctness.

5.2 Performance Measures

5.2.1 Accuracy

Accuracy measures the percentage of correctly classified cricket shots by the model.

Accuracy = (Number of Correct Predictions / Total Predictions) × 100

The model achieved an accuracy of approximately 84%, indicating good performance.

5.2.2 Confidence Score

The confidence score represents the probability with which the model predicts a particular shot. Higher confidence values indicate more reliable predictions.

5.2.3 Response Time

Response time refers to the time taken by the system to process each frame and provide output. The system performs in real time with minimal delay, making it suitable for live feedback.

5.2.4 Feedback Accuracy

Feedback accuracy measures how correctly the system suggests improvements based on joint angle comparison. The system effectively identifies posture errors and provides meaningful corrections.

5.3 Performance Analysis

The Random Forest model performs efficiently in classifying cricket shots based on pose keypoints. The use of pose-based features ensures robustness against background variations.

The implementation of prediction smoothing improves stability by reducing fluctuations in real-time predictions. The feedback system successfully compares user posture with ideal angles and provides corrective suggestions. The scoring mechanism helps users track their performance quantitatively.

| Metric | Value |
| --- | --- |
| Model Used | Random Forest |
| Number of Classes | 9–10 Shots |
| Training Accuracy | 84% |
| Input Type | Pose Keypoints |
| Real-Time Capability | Yes |

Table 5.1: Model Performance

5.4 Summary

This chapter evaluated the performance of the Smart Cricket system. The model achieved an accuracy of 84% and demonstrated effective real-time performance. The next chapter presents the conclusion and future enhancements of the system.

CHAPTER 6: FUTURE ENHANCEMENT AND CONCLUSION

6.1 Introduction

This chapter presents the limitations of the current Smart Cricket system, discusses possible future enhancements, and concludes the overall work carried out in this project.

The system demonstrates the application of artificial intelligence and computer vision in sports training, specifically for cricket batting analysis.

6.2 Limitations and Constraints of the System

Despite achieving good performance, the system has certain limitations:

- Dependence on lighting conditions: Poor lighting can affect pose detection accuracy

- Limited dataset: The number of cricket shots and training samples is limited

- Restricted joint analysis: Only elbow and knee angles are considered

- Camera dependency: Performance depends on camera quality and positioning

- No ball tracking: The system does not analyze ball trajectory

- Generalized feedback: Feedback is based on predefined ideal values and may not adapt to all players

6.3 Future Enhancements

The system can be further improved in the following ways:

- Use of Deep Learning Models: Implement CNN or LSTM for better accuracy

- Expanded Dataset: Include more cricket shots and diverse player data

- Advanced Pose Analysis: Analyze additional joints such as shoulder, hip, and wrist

- Mobile Application Integration: Develop a mobile-based version for easy access

- Voice Feedback System: Provide audio-based coaching instructions

- Ball Tracking Integration: Combine batting analysis with ball trajectory tracking

- 3D Visualization: Improve understanding using advanced 3D models

- Personalized Coaching: Adapt feedback based on individual player style

6.4 Conclusion

The project titled “Smart Cricket: AI-Based Batting Shot Detection and Real-Time Feedback System” successfully demonstrates the use of artificial intelligence in sports training.

The system effectively detects cricket shots using pose estimation and a Random Forest classifier with an accuracy of approximately 89%. It provides real-time feedback by comparing user posture with ideal professional standards, helping players improve their batting technique.

The integration of computer vision, machine learning, and real-time feedback makes this system a powerful and cost-effective virtual coaching solution. The project highlights the potential of AI in transforming traditional sports training methods and opens opportunities for further advancements in this field.

APPENDIX A

SAMPLE CODE SNIPPET

import cv2

import mediapipe as mp

import numpy as np

from collections import deque

mp_pose = mp.solutions.pose

pose = mp_pose.Pose()

cap = cv2.VideoCapture(0)

while cap.isOpened():

ret, frame = cap.read()

if not ret:

break

image = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = pose.process(image)

if results.pose_landmarks:

for landmark in results.pose_landmarks.landmark:

print(landmark.x, landmark.y)

cv2.imshow('Smart Cricket', frame)

if cv2.waitKey(10) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

APPENDIX B

DATASET DESCRIPTION

The dataset used in this project consists of cricket shot videos collected from professional players. The following shots were included:

- Cover Drive

- Straight Drive

- Pull Shot

- Hook Shot

- Sweep Shot

- Helicopter Shot

- Paddle Sweep

- Upper Cut

- Reverse Sweep

Frames were extracted from videos, and pose landmarks were generated using MediaPipe. Each frame was labeled according to the shot type.

REFERENCES

- B. Lugaresi et al., “MediaPipe: A Framework for Building Perception Pipelines,” arXiv preprint arXiv:1906.08172, 2019.

- L. Breiman, “Random Forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, 2001.

- G. Bradski, “The OpenCV Library,” Dr. Dobb’s Journal of Software Tools, 2000.

- R. Szeliski, Computer Vision: Algorithms and Applications, Springer, 2010.

- J. Shotton et al., “Real-Time Human Pose Recognition in Parts from Single Depth Images,” IEEE Conference on Computer Vision and Pattern Recognition, 2011.

- T. Chen and C. Guestrin, “XGBoost: A Scalable Tree Boosting System,” Proceedings of the ACM SIGKDD, 2016.

- Scikit-learn Developers, “Scikit-learn: Machine Learning in Python,” Available: https://scikit-learn.org

- Google Developers, “MediaPipe Pose Documentation,” Available: https://developers.google.com/mediapipe